Belt Obstacle Detection & Avoidance

Briefing notes for supervisor / professor presentation

Scope: feelSpace naviBelt guidance while walking toward a bottle; optional static chair in the path.
Code: hans_android/server/feedback_devices/adapters/belt_adapter.py
Branch: feat/belt_obstacle_avoidance

# 1. What problem does this solve?

Blind / low-vision users are guided toward a target bottle using waist vibration on a feelSpace naviBelt. Two experimental conditions use one shared codebase:

| Condition | Scene | What the belt does |
| --- | --- | --- |
| Clear path | Bottle only | Steer directly to the bottle |
| Obstacle | Bottle + one static chair between user and bottle | Guide around the chair, then resume approach to the bottle |

Experiment assumptions (important):

The only obstacle is a chair.

The chair does not move during a trial.

The bottle stays fixed.

No other obstacle types are expected.

At close range (~50 cm), the belt stops and hands off to a bracelet for grasping (separate work).

# 2. High-level system role

Phone (ARCore depth + YOLO bottle) → Server / BeltAdapter → feelSpace naviBelt (16 motors around the waist).

Detection of the bottle: YOLO (object detection) + depth for distance.

Detection of the chair: not YOLO. The chair is found from the depth map in a forward “corridor” region of the image.

Steering cue: one vibration direction around the waist (bearing) + intensity related to distance.

Clear corridor: vibrate toward bottle.

Chair in corridor: vibrate toward a side waypoint, then toward the bottle.

# 3. How chair detection works

## 3.1 Forward corridor (region of interest)

Each frame, the system looks only at a narrow vertical strip in front of the user:

Horizontally: centered on the image (~±10% of width; wider once avoidance has started).

Vertically: mid-band of the image (~22%–55% of height).

Why this band?

Captures a typical chair seat / back in the walking view.

Excludes the floor (lower band), which would otherwise look like a “close obstacle” in depth.

Ignores the sky / far background.

┌─────────────────────────────┐
  │         (upper FOV)         │
  │      ┌───────────────┐      │
  │      │  CORRIDOR ROI │  ← mid-height strip (chair band)
  │      └───────────────┘      │
  │         (floor band)        │  ← intentionally ignored
  └─────────────────────────────┘

## 3.2 Depth test (is there a chair?)

Inside the corridor, depth pixels are classified as “chair-like” if they are:

1. Valid depth (not missing / absurd).

2. Not part of the bottle bounding box (so the target itself is not treated as an obstacle).

3. Closer than the bottle by a margin (default ≥ 40 cm in front of the target), and not farther than ~2 m.

If a large enough fraction of valid corridor pixels are “closer than the bottle,” the corridor is marked blocked. From those closer pixels the system estimates:

Obstacle center (image x) — where the chair sits left–right.

Obstacle depth — median depth of those pixels.

If the fraction is too small, the corridor is clear (no chair for navigation purposes).

## 3.3 Robustness against depth noise

ARCore depth can flicker. To avoid false avoidance on an empty path:

The chair must appear for several consecutive frames (~5) before avoidance steering starts (OBSTACLE_CONFIRM_FRAMES).

A one-frame glitch does not pull the user sideways.

Floor clutter alone does not trigger avoidance (wrong image band).

# 4. Plan freeze (static scene)

Because the chair and bottle do not move, the system does not replan every frame. After ~10 consistent frames (PLAN_LOCK_FRAMES), it locks one of two plans:

Trial starts → inspect corridor.

Clear for ~10 frames → LOCK direct approach → steer to bottle only; ignore late false chair detections.

Chair for ~5 frames → choose pass side (L/R) → if still consistent ~10 frames → LOCK avoid plan → steer around chair then to bottle; never flip pass side mid-trial.

| Locked mode | Meaning |
| --- | --- |
| direct | No chair in plan. Always steer to bottle. Late depth noise cannot switch to avoidance. |
| avoid | Chair confirmed. Pass side + chair geometry frozen. Walk the plan until corridor clears, then resume bottle. |

Why lock? Without locking, frame-to-frame depth jitter can flip “go left / go right” and confuse the user. For a fixed chair, locking is the correct model.

# 5. What happens when a chair is present (two phases)

## Phase A — Pass the chair

1. Confirm chair over consecutive frames.

2. Choose pass side: prefer the side toward the bottle (shortest path). If the bottle is almost directly behind the chair, pick the side with more free space in the corridor.

3. Compute a side waypoint: an image column ~20 cm past the chair on the chosen side (using assumed camera FOV).

4. Belt vibrates toward that waypoint (not toward the bottle yet).

## Phase B — Resume the bottle

1. Keep checking the corridor.

2. When the corridor stays clear for a sustained window (~30 frames), treat the chair as passed.

3. Switch steering back to the bottle.

4. Continue normal approach until handoff distance.

User ──► [chair] ──► bottle

Phase A: vibrate toward side of chair
Phase B: vibrate toward bottle
Close:   belt stops → bracelet grasp (~50 cm)

# 6. Clear-path behavior (same code, no chair)

When there is no chair:

1. Steer to the bottle with a smoothed bearing (reduces jitter from detection).

2. After stability, lock direct.

3. Intensity increases as the user gets closer.

4. At ~80 cm: pre-handoff cue (navel pulses).

5. At ≤50 cm: handoff cue; belt stops.

This is the same adapter used for obstacle trials — only the locked plan differs.

# 7. Navigation cues (belt events)

| Cue | When | Rough pattern |
| --- | --- | --- |
| Start | Approach begins | 3 navel pulses |
| Direction change | Coarse left/center/right flip | Short all-around pulse (rate-limited) |
| Continuous | While walking | Smooth vibration toward steer direction |
| Pre-handoff | ~80 cm from bottle | 4 navel pulses |
| Handoff | ≤50 cm | All-around pulse; belt stops |

Avoidance does not use a special “obstacle alert” pattern; the user feels the change of steering direction toward the side waypoint.

# 8. Design choices to emphasize in the talk

Depth corridor, not chair class labels — works for the thesis chair without depending on YOLO “chair” detection.

One static chair — plan freeze is intentional, not a missing feature.

One codebase for both conditions — clear-path and chair trials share logic; confirmation + direct lock prevent false avoidance.

Belt = walk; bracelet = grasp — obstacle work sits only in the approach phase.

User stays oriented — continuous directional vibration + discrete cues; no visual dependence.

# 9. What this system does not do

Does not track a moving chair or replan around a relocated obstacle mid-trial.

Does not handle multiple obstacles or other obstacle types (tables, people, etc.).

Does not classify “chair” with YOLO; it detects a closer depth mass in the corridor.

Does not navigate after handoff (bracelet takes over).

# 10. Suggested slide outline (10–12 min)

1. Goal: belt approach to bottle; optional static chair

2. Dual conditions, one codebase

3. Pipeline: depth + YOLO → BeltAdapter → naviBelt

4. Corridor ROI (diagram)

5. Depth rule: closer-than-bottle fraction

6. Confirmation + plan lock (why static chair matters)

7. Phase A / Phase B walk-through

8. Clear-path case (same code)

9. Cues + handoff to bracelet

10. Limitations & next steps (optional)

# 11. Key parameters (reference)

| Parameter | Typical value | Role |
| --- | --- | --- |
| Corridor half-width | ~10% of image | Forward walking path |
| Corridor vertical band | ~22%–55% height | Chair seat/back; skip floor |
| Closer-than-target margin | 0.40 m | Chair must be in front of bottle |
| Enter fraction | 0.22 | How filled corridor must be to call “blocked” |
| Confirm frames | 5 | Noise rejection before avoidance |
| Plan lock frames | 10 | Freeze direct vs avoid |
| Clear frames to exit | 30 | Sustained clear → resume bottle |
| Safe margin past chair | 0.20 m | Side waypoint offset |
| Handoff enter | 50 cm | Belt → bracelet |

# 12. One-sentence summary

We guide the user to a bottle with waist vibration; if a single static chair blocks the forward depth corridor, we lock a pass-side plan, steer around it, then resume bottle approach — and the same code stays on a direct approach when the corridor is clear.